Sec. 72-284. Minimum landscaping requirements.

The purpose and intent of this section is to encourage the preservation of existing trees and vegetation; to identify landscape standards and plant classifications; to reduce radiant heat from surfaces and conserve energy; to provide shade; to reduce wind and air turbulence; to minimize potential nuisances such as the transmission of noise, dust, odor, litter and glare of automobile headlights; to provide visual buffering and for the separation of spaces; to enhance the beautification of the county; to reduce the amount of impervious surface and stormwater runoff; to safeguard and enhance property values and protect public and private investment; and to protect the public health, safety and general welfare.

The following regulations shall apply where landscaped buffer or off-street parking areas are required:

(1)	
      All plant materials shall be Florida No. 1 or better grade, according to the current Grades and Standards for Nursery Plants, State of Florida, Florida Department of Agriculture and Consumer Services, and selected from the "Plant Material List" available from the growth and resource management department. All plant materials shall be chosen, spaced and planted according to the current Grades and Standards for Nursery Plants, Florida-friendly best management practices, the Volusia County Environmental Management Division's Habitat Planting Guidelines, and "right plant, right place" principles, to protect and preserve Volusia County's water resources, at least 50 percent of the required landscaping specified in subsections a and b, shall individually be of a native drought tolerant species chosen from the plant material list. The Zoning Enforcement Official (ZEO) has the authority to allow existing native vegetation to provide the necessary visual screening. Existing upland native vegetation shall be incorporated, where appropriate, into off-street parking areas and landscape buffers of a proposed development.

a.	
      Specific requirements:

1.	
      Canopy trees (Group A). Canopy tree species shall be a minimum of 1-½-inch caliper and six feet in overall height immediately after planting. Trees required along county thoroughfares shall be a minimum of two-inch caliper and eight feet in overall height immediately after planting, and planted a minimum of ten feet back from the right-of-way line.

2.	
      Mid-story trees (Group B). Mid-story tree species shall be a minimum of 1-½-inch caliper and six feet in overall height immediately after planting. Trees required along county thoroughfares shall be a minimum of two-inch caliper and eight feet in overall height immediately after planting, and planted along a line ten feet back from the right-of-way line.

3.	
      Understory trees and shrubs(Group C). Understory trees shall be a minimum 1-½-inch caliper and four feet in overall height and shrubs shall be a minimum of two feet in overall height, immediately after planting.

4.	
      Groundcovers, large grasses and wildflowers (Group D). Large grasses shall be a minimum of 18 inches in overall height immediately after planting. Ground covers and wildflowers do not have a minimum height requirement, as maximum height is dependent upon the specific plant species. At least 25 percent of the project area outside of any landscape buffers, off-street parking areas, and stormwater management areas, shall be planted and maintained with a drought tolerant ground cover(s). Ground covers must present a finished appearance and provide reasonably complete coverage at time of planting.

b.	
      Additional requirements:

1.	
      Palms. Palms shall constitute no more than 25 percent of the required canopy trees and shall have a minimum of six feet of clear wood at planting.

2.	
      Lawn grass. Grassed areas shall be planted with any species of grass common to Central Florida. They may be sodded, plugged, sprigged or seeded; except that sod is required in swales or other areas subject to soil erosion. Unless sod or grass seed is used, nursegrass seed shall be sown for immediate effect and protection until coverage is otherwise achieved. Grass sod shall be clean and shall be reasonably free of weeds, noxious pests or diseases.

(2)	
      Landscaped buffer area. Where a landscaped buffer area is required, the following regulations shall apply:

a.	
      A landscaped buffer area complying with the requirements of the landscape buffer tables (Table I, Table II, Table III, and Table IV), shall be established along the entire length of, and contiguous to, any property line. The width of the area, plant and tree quantities, and type of plant materials shall be determined from the landscape buffer requirements within Table I, Table II and Table III, and landscaped with the required mix of plant species, as determined in Table IV. A driveway or entranceway shall not be considered part of the landscape buffer for the purpose of calculating the required amount of plant material. Landscape buffer widths may be increased to include existing native trees and their associated drip line, and to provide aesthetically pleasing façades to proposed establishments. Compliance with Tables I through IV is achieved through the following steps:

1.	
      Determine the zoning classification of the premises and the adjoining properties.

2.	
      Utilize Table I to determine which landscape buffer zoning classification or special exception use group the zoning classification or use of the premises and the adjoining properties fall into.

3.	
      Determine the buffer type and screening requirements required between the subject group (subject premises) and adjoining groups (adjoining properties) in Table II. Required screen types are listed immediately after Table II.

4.	
      Determine the specific requirements (i.e. width and required plantings) for each buffer type in Table III by corresponding the required buffer types required by Table II.

5.	
      Determine the required number of plant species by utilizing Table IV.

6.	
      Example: The subject premises is within the B-4 General Commercial zoning classification, abuts a property within the R-4 single-family residential zoning classification along its 200 lineal-foot boundary. Pursuant to Table I, the subject premises is within the commercial group and the abutting property is within the one and two-family residential group. Table II states that a subject group of commercial adjoining the One and two-family residential group require Buffer Type 4 and Screen Type 2. Screen Type 2 is listed immediately after Table II, with additional screening requirements referenced in subsection 72-284(2)d. Table III states that Buffer Type 4 has a minimum 25-foot width and requires 3 Group A canopy trees, 5 Group B mid-story trees and 25 Group C understory trees/shrubs per 100 lineal feet. A 200 lineal-foot buffer length would then require a minimum of 6 Group A canopy trees, 10 Group B mid-story trees and 50 Group C understory trees/shrubs. Table IV would determine that 2 species are required for Group A plantings, 2 species are required for Group B plantings, and 7 species are required for Group C plantings.

b.	
      The landscaped buffer area shall contain existing native vegetation, which is not dependent on irrigation because of the possibility of water use restrictions, which could adversely affect the survival of introduced or exotic plant species. Existing native trees that are situated within the required buffer can be used to satisfy the perimeter tree requirement. The area(s) within the dripline(s) of the existing native trees must be included within the landscaped buffer area.

c.	
      New trees in required landscape buffers do not have to be equally spaced, but may be grouped to provide areas of visibility from the proposed site to an adjoining road and shall be located out of the driplines of existing specimen and historic trees. New shrubs in required landscape buffers shall also be located to minimize root zone encroachment, if applicable visual screening requirements of subsection 72-284(2)(d) are met.

d.	
      Required screening specified by Table III shall comply with the following standards:

1.	
      Brick or masonry walls and vinyl or wood fences utilized as visual screens must be opaque and finished to enhance site aesthetics.

2.	
      Wood fences shall meet the following requirements: Minimum ½-inch thick pressure treated boards, minimum three two-inch by four-inch pressure treated horizontal stringers, and minimum four-inch by four-inch-thick pressure treated support posts at eight-foot maximum spacing. Alternate fence materials may be used with the approval of the ZEO.

3.	
      Hedges, utilized as visual screens, shall be planted at a minimum four feet in overall height and separated at a distance of 30 inches or less. These hedges shall be planted and maintained to provide a six-foot high continuous, unbroken, solid, visual screen within a maximum of one year after date of planting.

4.	
      A new landscaped earthen berm may only be used in landscaped buffers 50 feet wide or larger. These berms shall be planted in accordance with the requirements of landscape buffer Tables I through IV. Landscaped earthen berms shall have a maximum slope of four feet horizontal to one-foot vertical (4:1) and shall include natural ground covers that provide at least 90 percent coverage of the entire berm area.

5.	
      When a wall, fence, or hedge is used for screening, existing native vegetation should be retained, or planted in accordance with the requirements of the landscape buffer tables (Table I, II, III and IV). At least 50 percent of the required plant materials shall be planted along the exterior side of the screen. The screen shall include an opaque maintenance gate every 100 lineal feet or half the length of the screen, whichever is less. The remaining plant materials shall be planted on the interior side of the screen to enhance the site's interior.

e.	
      A minimum of 50 percent of the required landscape buffer shall remain at natural grade or 6:1 slope, while the remaining buffer area shall not exceed a 4:1 slope.

f.	
      When a recorded subdivision consists of ten lots or more, the required landscape buffer area shall be platted as a separate tract of land and maintained by a homeowners or property owners association to ensure adequate maintenance of the buffer.

g.	
      Where canopy trees are required in a coastal environment, but appropriate salt tolerant species are not available, the required canopy trees can be substituted with mid-story trees at a ratio of two mid-story trees for every canopy tree, with approval of the zoning enforcement official (ZEO).

h.	
      Removal of vegetation categorized as an invasive exotic in the Florida Exotic Pest Plant Council's List of Invasive Species and debris is required within all landscaped buffers. Care shall be taken not to damage, destroy, or otherwise remove healthy, beneficial, native vegetation within these areas.

i.	
      The ZEO or the county council may require a greater buffer area than those listed in Tables II and III for a permitted principal or special exception use, due to specific site conditions in conjunction with the existing character of the surrounding area. The ZEO or county council shall determine the appropriate buffer width, required plantings and screening requirements, if applicable, for special exception uses not listed in the aforementioned tables. In no case shall the required landscaped buffer be less than 25 feet between a nonresidential and residential use.

j.	
      The ZEO may waive or modify the requirements of subsection 72-284(2), in order to facilitate the redevelopment of existing sites or where environmental constraints (i.e. preservation of existing wetlands, trees or topography) limit the available area to reasonably develop, or if literal interpretation of subsection 72-284(2) creates a hardship. The following items are prohibited within the required perimeter landscaped buffers:

1.	
      A public or private right-of-way, easement, structure, or impervious surface other than a sign, an entrance or a driveway, or a site specific water or sanitary sewer service line which traverses the landscaped buffer.

2.	
      Stormwater management areas, and any associated easements and appurtenances.

3.	
      Private or public utility easements, structures or above-ground and underground improvements, except as stated above.

| TABLE I Landscape Buffer Zoning Classification Groups |  |
| --- | --- |
| Group | Specific Zoning Classifications or Special Exception Uses |
| Agricultural and Conservation | C, FR, RC, A-1 through A-4, and MH-3 |
| Airport and Public | AP and P |
| Commercial | B-1 through B-9, BPUD, MPUD (commercial areas) |
| Industrial | I-1 through I-4, IPUD and MPUD (industrial area) |
| One and two-family residential | RR, RA, R-1 through R-9, RPUD, and MPUD (residential area) |
| Mobile home | MH-4 through MH-6, MH-8, RPUD, and MPUD (residential area) |
| Mobile home park | MH-1, MH-2 and MH-7 |
| Multifamily residential | R-6 (with multifamily special exception), R-7, R-8, RPUD, and MPUD (residential area) |
| Special Exception Use Group 1 | Air curtain incinerator Asphalt/Cement batching plant - permanent or temporary Construction and demolition debris disposal facility Hazardous waste transporter facility Junkyard Landfill Materials recovery facility Nonexempt excavation Off-site disposal of land clearing debris facility Private utility uses and services Public utility uses and services Solid waste transfer station Recovered materials facility Rock crusher |
| Special Exception Use Group 2 | Animal hospital Animal shelters Dogs and cats boarded for personal pets exceeding the number permitted in subsection 72-306(a) Kennel Veterinary clinic |
| Special Exception Use Group 3 | Fixed-wing aircraft landing fields and helipads Heliports and helipads accessory to a principal use Recreational areas and similar uses (residential and commercial) Speedways, racetracks and motorized vehicle motocross courses |
| Special Exception Use Group 4 | Mini-warehouse Flea market Off-street parking areas Parking garages Processing, packaging, storage, retail or wholesale sales of agricultural products not raised on the premises |
| Special Exception Use Group 5 | Boarding house Cemetery Day care center Group home Home Occupations Class B Nursing home Private clubs Schools parochial, private, professional or trade |
| Rural Event Special Exception Use | Rural Event Center |

| TABLE II Landscape Buffer Requirements |  |  |  |
| --- | --- | --- | --- |
| Subject Group | Adjoining Group | Buffer Type | Screening Required* |
| Agricultural and Conservation | Agricultural and Conservation | 2 | Yes - Screen Type 1 if subdivision is over 10 lots |
|  | Airport and Public | 5 | Yes - Screen Type 2 if subdivision is over 10 lots |
|  | Commercial | 4 | Yes - Screen Type 2 if subdivision is over 10 lots |
|  | Industrial | 4 | Yes - Screen Type 2 if subdivision is over 10 lots |
|  | One and two-family residential | 3 | Yes - Screen Type 1 if subdivision is over 10 lots |
|  | Mobile home | 3 | Yes - Screen Type 1 if subdivision is over 10 lots |
|  | Mobile home park | 3 | Yes - Screen Type 1 if subdivision is over 10 lots |
|  | Multifamily residential | 3 | Yes - Screen Type 1 if subdivision is over 10 lots |
| Airport and Public** | Agricultural and Conservation | 5 | Yes - Screen Type 2 |
|  | Airport and Public | 1 | No |
|  | Commercial | 1 | No |
|  | Industrial | 1 | No |
|  | One and two-family residential | 6 | Yes - Screen Type 3 |
|  | Mobile home | 6 | Yes - Screen Type 3 |
|  | Mobile home park | 6 | Yes - Screen Type 3 |
|  | Multifamily residential | 6 | Yes - Screen Type 3 |
| Commercial | Agricultural and Conservation | 4 | Yes - Screen Type 2 |
|  | Airport and Public | 1 | No |
|  | Commercial | 1 | Optional - Screen Type 2 if required by ZEO or county council |
|  | Industrial | 1 | Yes - Screen Type 2 |
|  | One and two-family residential | 4 | Yes - Screen Type 2 |
|  | Mobile home | 4 | Yes - Screen Type 2 |
|  | Mobile home park | 4 | Yes - Screen Type 2 |
|  | Multifamily residential | 4 | Yes - Screen Type 2 |
| Industrial | Agricultural and Conservation | 4 | Yes - Screen Type 2 |
|  | Airport and Public | 1 | Yes - Screen Type 2 |
|  | Commercial | 1 | Yes - Screen Type 2 |
|  | Industrial | 1 | Optional - Screen Type 2 if required by ZEO or county council |
|  | One and two-family residential | 5 | Yes - Screen Type 3 |
|  | Mobile home | 5 | Yes - Screen Type 3 |
|  | Mobile home park | 5 | Yes - Screen Type 3 |
|  | Multifamily residential | 5 | Yes - Screen Type 3 |
| One and two-family residential | Agricultural and Conservation | 3 | Yes - Screen Type 1 if subdivision is over 10 lots |
|  | Airport and Public | 6 | Yes - Screen Type 2 |
|  | Commercial | 4 | Yes - Screen Type 2 |
|  | Industrial | 5 | Yes - Screen Type 2 |
|  | One and two-family residential | 2 | Yes - Screen Type 1 if subdivision is over 10 lots |
|  | Mobile home | 2 | Yes - Screen Type 2 if subdivision is over 10 lots |
|  | Mobile home park | 3 | Yes - Screen Type 2 |
|  | Multifamily residential | 3 | Yes - Screen Type 2 |
| Mobile home | Agricultural and Conservation | 3 | Yes - Screen Type 1 if subdivision is over 10 lots |
|  | Airport and Public | 6 | Yes - Screen Type 3 |
|  | Commercial | 4 | Yes - Screen Type 2 |
|  | Industrial | 5 | Yes - Screen Type 3 |
|  | One and two-family residential | 2 | Yes - Screen Type 2 |
|  | Mobile home | 2 | Yes - Screen Type 1 if subdivision is over 10 lots |
|  | Mobile home park | 3 | Yes - Screen Type 1 if subdivision is over 10 lots |
|  | Multifamily residential | 3 | Yes - Screen Type 3 |
| Mobile home park | Agricultural and Conservation | 3 | Yes - Screen Type 3 |
|  | Airport and Public | 6 | Yes - Screen Type 3 |
|  | Commercial | 4 | Yes - Screen Type 3 |
|  | Industrial | 5 | Yes - Screen Type 3 |
|  | One and two-family residential | 3 | Yes - Screen Type 3 |
|  | Mobile home | 3 | Yes - Screen Type 3 |
|  | Mobile home park | 2 | Yes - Screen Type 1 |
|  | Multifamily residential | 3 | Yes - Screen Type 3 |
| Multifamily residential | Agricultural and Conservation | 3 | Yes - Screen Type 2 |
|  | Airport and Public | 6 | Yes - Screen Type 3 |
|  | Commercial | 4 | Yes - Screen Type 2 |
|  | Industrial | 5 | Yes - Screen Type 3 |
|  | One and two-family residential | 3 | Yes - Screen Type 3 |
|  | Mobile home | 3 | Yes - Screen Type 3 |
|  | Mobile home park | 3 | Yes - Screen Type 2 |
|  | Multifamily residential | 2 | Optional - Screen Type 2 if required by ZEO or county council |
| Special Exception Use Group 1 | All groups | 6 | Yes - Screen Type 4 |
| Special Exception Use Group 2 | Agricultural and Conservation | 3 | Yes - Screen Type 3 |
|  | Airport and Public | 2 | No |
|  | Commercial | 2 | No |
|  | Industrial | 2 | No |
|  | One and two-family residential | 4 | Yes - Screen Type 3 |
|  | Mobile home | 4 | Yes - Screen Type 3 |
|  | Mobile home park | 4 | Yes - Screen Type 3 |
|  | Multifamily residential | 4 | Yes - Screen Type 3 |
| Special Exception Use Group 3 | Agricultural and Conservation | 5 | Yes - Screen Type 3 |
|  | Airport and Public | 2 | No |
|  | Commercial | 2 | No |
|  | Industrial | 2 | No |
|  | One and two-family residential | 5 | Yes - Screen Type 4 |
|  | Mobile home | 5 | Yes - Screen Type 4 |
|  | Mobile home park | 5 | Yes - Screen Type 4 |
|  | Multifamily residential | 5 | Yes - Screen Type 4 |
| Special Exception Use Group 4 | Agricultural and Conservation | 3 | Yes - Screen Type 3 |
|  | Airport and Public | 1 | No |
|  | Commercial | 1 | No |
|  | Industrial | 1 | No |
|  | One and two-family residential | 4 | Yes - Screen Type 3 |
|  | Mobile home | 4 | Yes - Screen Type 3 |
|  | Mobile home park | 4 | Yes - Screen Type 3 |
|  | Multifamily residential | 4 | Yes - Screen Type 3 |
| Special Exception Use Group 5 | Agricultural and Conservation | 5 | Yes - Screen Type 2 |
|  | Airport and Public | 3 | No |
|  | Commercial | 2 | No |
|  | Industrial | 3 | Yes - Screen Type 2 |
|  | One and two-family residential | 5 | Yes - Screen Type 3 |
|  | Mobile home | 5 | Yes - Screen Type 3 |
|  | Mobile home park | 5 | Yes - Screen Type 3 |
|  | Multifamily residential | 5 | Yes - Screen Type 3 |
| Rural Even Center Special Exception | All groups | Provide Buffer Type 3; 20-foot landscape buffers between any rural event center facility or any area use for an event, including, but not limited to, parking areas, street frontages and adjacent properties | Optional - Screen Type RE if required by the ZEO or county council |
| All groups | All public uses in all zoning classifications | 3 | Yes - Screen Type P |
| All groups | Houses of worship*** | 3 | Yes - Screen Type 2 |
| All groups | Thoroughfare road rights-of-way as identified in the Volusia County Comprehensive Plan | 3 | As identified in applicable screen requirements |
| All groups | Non-thoroughfare road rights-of-way | 2 | As identified in applicable screen requirements |
| All groups | Railroad rights-of-way | 3 | No |

*Screening shall meet the requirements of subsection 72-284(2)d.

**Landscaping shall not be required adjacent to airside operations.

***Houses of worship shall meet Buffer Type 5 if adjoining the Airport/Public group.

Screen Type 1: Except for property street frontages, provide a continuous 4-foot-high visual screen of landscape planting hedge from Group C of the plant material list, or a finished brick or masonry wall, or wooden fence, or combination of the preceding screens, along and adjacent to all common boundaries.

Screen Type 2: Except for property street frontages, provide a continuous 6-foot-high visual screen of landscape planting hedge from Group C of the plant material list, or a finished brick or masonry wall, or wooden fence, or landscaped earth berm, or combination of the preceding screens, along and adjacent to all common boundaries. Landscaped earth berms are only permitted in Buffer Type 6 pursuant to subsection 72-284(2)d.

Screen Type 3: In property street frontages adjacent to parking areas, provide a continuous 3-foot-high finished brick or masonry wall or landscaped earth berm or wooden fence or combination of the preceding screens. In addition, along all other common boundaries, provide a continuous 6-foot-high visual screen of landscape planting hedge from Group C of the plant material list, a finished brick or masonry wall, or wooden fence, or landscaped earth berm, or combination of the preceding screens. Landscaped earth berms are only permitted in Buffer Type 6 pursuant to subsection 72-284(2)d.

Screen Type 4: Adjacent to all property boundaries, including street frontages, provide a continuous 6-foot-high finished brick or masonry wall or wooden fence or landscaped earth berm, or combination of the preceding unless existing vegetation serves as adequate screening within the required buffer.

Screen Type RE: Screens are optional. Screens if required, shall be provided in or adjacent to required landscape buffer(s) and located between any rural event center or any area used for an event, including but not limited to, parking areas, street frontages and adjacent properties. Screens, if required, shall consist of a continuous 6-foot-high visual screen of landscape planting hedge from Group C of the plant material list; or a finished brick or masonry wall, or wooden fence.

Screen Type P: In property street frontages adjacent to parking areas, provide a continuous 3-foot-high finished brick or masonry wall or wooden fence or combination of the preceding screens. In addition, along other property boundaries, provide a continuous 6-foot-high visual screen of landscape planting hedge from Group C of the plant material list, a finished brick or masonry wall, or wooden fence, or combination of the preceding screens, along and adjacent to all common boundaries.

| TABLE III Required Landscape Buffer Types, Widths and Plantings |  |  |
| --- | --- | --- |
| Buffer Type | Minimum Buffer Width | Minimum Required Plantings* |
| 1 | 10 feet | 5 Group B Mid-Story Trees and 15 Group C Understory Trees/Shrubs |
| 2 | 15 feet | 1 Group A Canopy Tree, 3 Group B Mid-Story Trees and 20 Group C Understory Trees/Shrubs |
| 3 | 20 feet | 2 Group A Canopy Trees, 4 Group B Mid-Story Trees and 25 Group C Understory Trees/Shrubs |
| 4 | 25 feet | 3 Group A Canopy Trees, 5 Group B Mid-Story Trees and 25 Group C Understory Trees/Shrubs |
| 5 | 30 feet | 4 Group A Canopy Trees, 6 Group B Mid-Story Trees and 25 Group C Understory Trees/Shrubs |
| 6 | 50 feet | 5 Group A Canopy Trees, 7 Group B Mid-Story Trees and 25 Group C Understory Trees/Shrubs |

*The required plantings are per 100 lineal feet, or fraction thereof.

| TABLE IV Required Number of Minimum Plant Species |  |
| --- | --- |
| Required number of plant materials (per Group A, B, and C plants) | Minimum number of species |
| 1 - 10 | 2 |
| 11 - 15 | 3 |
| 16 - 20 | 4 |
| 21 - 25 | 5 |
| 26 - 30 | 6 |
| Over 30 | 7 |

(3)	
      Landscaping of off-street parking areas.

a.	
      Required vehicular use areas having off-street parking spaces for more than four vehicles shall have interior landscaped areas, excluding any required landscaped buffer areas. A minimum of 35 square feet of landscaping for each parking space shall be provided within the interior of an off-street parking area. A portion of the interior landscaped area shall be located at the ends of each row of interior parking spaces not abutting the perimeter of the parking area. Landscaped islands shall be dispersed within rows of contiguous parking spaces so that there is at least one landscaped island for every ten contiguous parking spaces within the row. Landscaped row ends and landscaped islands shall have a minimum area of 175 square feet with no width less than ten feet measured inside of curb and no length less than 17.5 feet if it abuts one parking space, or 35 feet if it abuts two parking spaces. All landscaped row ends and landscaped islands shall be depicted on the site plan with dimensions measured from inside of curb to inside of curb and total area. Two feet of these interior landscaped areas may be part of the required depth of each abutting parking space, provided curbs are used to protect them.

b.	
      Continuous non-mountable six-inch high concrete curbing shall be designed to standard specifications and installed along the edges of all landscape areas adjacent to vehicular use areas. The ZEO may approve an acceptable alternative material or method, such as the use of wheel stops, in order to facilitate the development of an alternative material off-street parking area.

c.	
      Each interior landscaped area shall include at least one mid-story tree. Canopy trees are only permitted in landscaped islands or row ends with a minimum 500 square feet in area. If shade trees are required for parked cars per subsection 72-303(g)(8), mid-story trees can be utilized. The remaining area shall be landscaped with at least 50 percent shrubs or a mix of suitable groundcovers, large grasses or wildflowers, which are less than four feet high at maturity. Other materials, such as stone, gravel or mulch may be used with the aforementioned plant materials.

d.	
      The ZEO may approve alternative designs utilizing decorative architectural planters in lieu of interior landscaped areas, in order to facilitate the redevelopment of existing sites or where environmental constraints (i.e. preservation of existing wetlands, trees or topography) limit the available area to develop. Such architectural planters shall contain a surface area of not less than 10 square feet and shall have a minimum interior width of 15 inches.

e.	
      Where the strict application of this subsection will seriously limit the function of the parking area, the required landscaping may be located near the perimeter of the paved area, with approval of the ZEO.

(4)	
      Foundation plantings.

a.	
      A minimum five-foot wide foundation planting area shall be provided and maintained adjacent to all building facades designed for public view or access. Foundation plantings shall include a combination of drought tolerant shrubs and large grasses. All landscape plant material type, size, and spacing, shall be consistent with the requirements of subsection 72-284(1). Each foundation planting area shall be landscaped as follows:

1.	
      One understory tree shall be required for every 30 lineal feet of the exterior building façade.

2.	
      One foundation plant shall be required for every one-lineal foot of the exterior building façade.

3.	
      Groundcovers and wildflowers are required and shall be maintained along and adjacent to each on-site pedestrian walkway to enhance the site's aesthetics.

b.	
      The ZEO may approve the reduction or elimination of foundation plantings in order to facilitate the redevelopment of existing sites or where environmental constraints (i.e. preservation of existing wetlands, trees or topography) limit the available area to develop.

(5)	
      Additional requirements. Premises located in the following special planning areas must comply with additional landscaping requirements. In cases of conflict, the most stringent requirements apply.

a. 1. Thoroughfare Overlay Zone: Subsections 72-297(d) and (e).

2.	
      Nonresidential Development Design Standards: Subsections 72-303(g) and (i).

3.	
      Enterprise Community Overlay Zone: division 1, section 72-1301 and subsection 72-1303(11), Overlay Zones [article IV].

4.	
      Osteen Local Plan: division 2, section 72-1309, Overlay Zones [article IV].

5.	
      DeLeon Springs Business Corridor: division 3, section 72-1352, Overlay Zones [article IV].

6.	
      Southwest Activity Center: division 5, section 72-1369, Overlay Zones [article IV].

(6)	
      Irrigation system plan.

a.	
      A workable underground irrigation system shall be installed in any area required to be landscaped. An irrigation plan shall be submitted which specifies sprinkler head type, pipe size, radius of throw, valve and backflow preventer location, location of well or source of water and other relevant information for an irrigation system. If an automatic sprinkler system is used, a rain sensor device or switch that will override the irrigation cycle when adequate rainfall has been accrued shall be installed as required by F.S. § 373.62.

b.	
      The irrigation system shall be fully operational and shall be operated on a regular basis. In situations where drought-resistant plant materials have not been properly maintained primarily due to lack of sufficient watering, the ZEO may require the installation of a permanent irrigation system or other irrigation methods meeting the specifications of this section. Compliance with the standards of the Volusia County Water Wise Ordinance, as amended, is required.

c.	
      Where an effluent re-use system is available to serve the premises, and sufficient capacity exists, then reclaimed water from said system shall be used to irrigate any area required to be landscaped in lieu of using potable water.

d.	
      In the interest of conserving the public's diminishing water resources and the promotion of Florida-friendly landscaping principles, the ZEO may waive the automatic irrigation system requirement, in lieu of an acceptable alternative, such as the use of a low-volume drip emitter, porous pipe or similar means based on the review of the landscape plan. Landscaped areas containing primarily native drought tolerant species may provide a temporary irrigation system for a minimum of two years after initial installation, with approval of the ZEO. Once the native landscape materials have been firmly established, the temporary irrigation system may be removed.

(7)	
      Landscape plan. When landscaped areas are required by this article, a scaled and labeled landscape plan prepared by and signed and sealed by a Florida-registered professional, including by not limited to a Florida licensed landscape architect, shall be submitted. The plan shall indicate the type, size, species, quantity, quality grades, common names, and scientific names of all existing and proposed plant materials in a written plant schedule. The location of all plant materials shall be identified on the plan with graphic symbols and callouts. The landscape plan shall identify all landscape buffer areas, vehicular use areas, dimensions, the driplines of existing native trees to be preserved, and scaled, dimensioned and labeled property lines. The proposed and total number(s) of all plant materials in each buffer, each required landscape area, and in each vehicular use area shall be provided. A written statement of how these numbers were calculated is also required. The plan shall be reviewed by the growth and resource management department, and no development order shall be issued until the plan is approved. Any person aggrieved by a determination of the ZEO under this section may appeal that determination as provided in section 72-378.

(8)	
      Landscape installation. A Florida licensed landscape architect shall submit to the county a dated, signed and sealed affidavit of completion that verifies the landscaping has been installed according to the approved plan(s). The affidavit shall specifically indicate that the required plantings were installed as specified by the approved landscape plan and per Florida Grades and Standards, that all invasive or exotic species and debris were removed, that required site irrigation was installed as designed, and that an on-site irrigation audit has been performed. A final inspection shall not be passed for any site development projects until the landscape affidavit of completion is received, accepted and a site inspection occurs confirming compliance with all approved plans, as determined by the ZEO.

(9)	
      Maintenance. Where landscaped areas are required by this article, the owner, tenant or agent, of said entity will be responsible for maintenance of all required landscape and irrigation improvements, including the removal of invasive and exotic species and debris, as originally approved, in perpetuity.

Landscape areas and site improvements shall be maintained in good condition for a healthy, neat and orderly appearance and shall be kept free from weeds and debris. All plant materials shall be maintained in a healthy and vigorous condition through proper irrigation, fertilization, pruning, weeding, mowing, and other standard horticultural practices. Plant material should grow to their normal shape, color and height, to fulfill the required functions of screening, shading, buffering and aesthetic appeal. The hatracking of trees is prohibited. All dead plants shall be replaced. This requirement includes, but is not limited to, the replacement of plants damaged by insects, diseases, vehicular traffic, acts of God and vandalism. The county shall notify the property owner in writing of any maintenance violations.

(10)	
      Use of drought-resistant plants. All new or replacement plantings required for any off-street parking area or landscape buffer shall use, to the fullest extent possible, native plant material or other species with equivalent drought-resistant properties. Salt tolerant plants and trees as identified on the plant material list are required to be used on-sites within the coastal environment. It is the intent of this requirement to promote the conservation of the county's water resources to be consistent with the Florida-friendly principles of F.S. § 125.568.

(11)	
      Solid waste containers.

a.	
      All solid waste containers, except approved recycling containers, shall be enclosed on at least three sides with a six-foot high screen, and include an opaque access gate. The screen shall consist of an opaque brick or finished masonry wall, vinyl or pressure treated wood fence. The ZEO may require that a hedge or similar landscaping material be provided and maintained along the enclosure walls.

b.	
      The container shall be enclosed in such a manner so that said container will be screened from public streets and adjoining properties. A concrete or asphalt pad of appropriate size and construction shall be provided as a base for the container. The container pad shall be at the approximate level of the service vehicle approach area so that the truck's loading mechanism can align with the container's sleeves.

c.	
      The screened enclosure shall not be located within any street right-of-way or required yard area. Containers and enclosures shall be located to allow ease of access for collection trucks and direct access to drive areas. Straight-in or circular drives are encourage to reduce truck maneuvering problems. No parking or other obstructions shall be permitted in the access area for enclosures.

Editor's note(s)—Prior to its reenactment by § 59 of Ord. No. 90-24, adopted Sept. 27, 1990, subsection 808.07, "Guarantee of Installation of Improvements," was repealed by Ord. No. 88-2, § IX, adopted Jan. 19, 1988.

(12)	
      Overhead electrical utilities. If the buffer area contains or will contain overhead electrical wires, or if existing or proposed overhead electrical wires are within 30 feet of the buffer area, tree selection shall be limited to those trees that will not, at mature height, conflict with overhead utilities. Canopy trees (height at maturity of > 30 feet) shall be planted no closer than a horizontal distance of 30 feet from the nearest existing or proposed overhead conductor. Mid-story trees (height at maturity between 20 and 30 feet) shall be offset at least 20 feet and understory trees (height at maturity of < 20 feet) require no offset. Palms shall be planted no closer than a horizontal distance equal to the average mature frond length plus two feet from the nearest existing or proposed conductor. Climbing vines shall not be planted adjacent to utility poles, transformers, or guy wires. Plantings near pad-mounted transformers shall not restrict access to, or the maintenance of, the transformer.

a.	
      If overhead electrical utilities impact the ability to utilize the required number of canopy or mid-story tree plantings, as stated within Tables I, II and III, the ZEO may allow substitution of understory trees or shrubs for required canopy and/or mid-story trees.

(Ord. No. 81-39, § XL, 11-19-81; Ord. No. 84-1, § 6, 3-8-84; Ord. No. 85-24, §§ XIV, XV, 10-10-85; Ord. No. 86-16, §§ XVII—XIX, 10-23-86; Ord. No. 87-14, §§ XI—XIV, 6-18-87; Ord. No. 89-20, §§ XXVI—XXVIII, 6-20-89; Ord. No. 90-34, §§ 54—60, 9-27-90; Ord. No. 91-11, § XIV, 5-16-91; Ord. No. 92-6, § XLIX, 6-4-92; Ord. No. 94-4, §§ LXII, LIX—LXI, LXIV, LXV, 5-5-94; Ord. No. 98-25, §§ XIX—XXIV, 12-17-98; Ord. No. 2004-20, § V, 12-16-04; Ord. No. 2005-02, § III, 4-21-05; Ord. No. 2008-25, § II, 12-4-08; Ord. No. 2013-08, § III, 5-2-13; Ord. No. 2018-05, § XXIV, 1-18-18; Ord. No. 2022-1, II(Exh. B), 1-18-22)