ORDINANCE 2025-

AN ORDINANCE OF THE COUNTY COUNCIL OF VOLUSIA COUNTY, FLORIDA, RELATING TO THE LAND DEVELOPMENT CODE OF VOLUSIA COUNTY; AMENDING CHAPTER 72, ARTICLE III DIVISION 8, SECTION 72-778 STORMWATER MANAGEMENT PERMIT REVIEW TO INCLUDE SOIL BORINGS; AMENDING CHAPTER 72, ARTICLE III, DIVISION 8, SECTION 72-779 PERFORMANCE, REVIEW, AND DESIGN STANDARDS TO INCLUDE SEASONAL HIGH-WATER ELEVATION AND BASIN STUDY; PROVIDING FOR SEVERABILITY; PROVIDING FOR CONFLICTING ORDINANCES; AND PROVIDING FOR AN EFFECTIVE DATE.

WHEREAS, Chapter 163, Part II, of Florida Statutes, entitled the Local Government Comprehensive Planning and Land Development Regulations Act ("Act"), empowers and requires the Volusia County Council (the "Council") to plan for the County's future development and growth and to adopt and amend its Land Development Code, or elements of portions thereof, to guide the future growth and development of the County; and

WHEREAS, the Volusia County Council has a goal of updating the Land Development Code to promote consistency with the latest state and federal laws, as well as best practices for land development in the County; and

WHEREAS, the County Council of Volusia County, Florida has determined that the proposed amendment serves the public health, safety and welfare of the citizens of Volusia County.

WHEREAS, this ordinance has met the notice and public hearing requirements of 43 sections 125.66(4)(b), F.S., and was heard before the Planning and Land Development 44 Regulation Commission pursuant to section 163.3174, F.S.

BE IT ORDAINED BY THE COUNTY COUNCIL OF VOLUSIA COUNTY, FLORIDA, AS FOLLOWS:

(Words in strike through type are deletions; words in underscore type are additions.)

SECTION I:  Incorporation of Recitals. The above recitals represent the legislative findings of the Volusia County Council supporting the need for this ordinance.

SECTION II: Chapter 72, Article III, Division 8, Section 72-778 of the Land Development Code of Volusia County, is hereby amended to read as follows:

ARTICLE III. – LAND DEVELOPMENT REGULATIONS

…

DIVISION 8. – STORMWATER MANAGEMENT

…

Sec. 72-778. - Stormwater management permit review.

…

An application for a stormwater management development permit shall be filed, processed and approved in the following manner:

…

(3) Stormwater management requirements.

…

c. The following plans and information, prepared by a Florida registered engineer, shall be submitted with the application:

…

- Information regarding the types of soils and groundwater conditions existing on the site, including a geotechnical investigation report signed by an engineer or geologist registered in the State of Florida and experienced in soils, hydrogeology and groundwater hydrology and an evaluation of seasonal high-water table elevations which contains:

  - A representative number of soil boring profiles, but not less than a minimum of two soil borings per acre within the footprint of the final pond location;

  - Depth measurements to the water table for each soil boring profile;

…

- If submitted estimates of seasonal high groundwater elevations differ by more than six inches from the values published in the official soil survey of Volusia County, Florida, additional evaluations explaining the discrepancy are required. The following additional information may also be required at the discretion of the CDE county development engineer:

…

SECTION III: Chapter 72, Article III, Division 8, Section 72-779 of the Land Development Code of Volusia County, is hereby amended to read as follows:

…

Sec. 72-779. - Performance, review and design standards.

- Performance Standards.

  - For applications for a lesser or a standard development, the following performance standards shall be followed in the design of the project:

    - Stormwater runoff shall be subjected to best management practice prior to discharge into natural or artificial drainage systems. "Best management practice" shall mean a practice or combination of practices determined by the DRC to be the most effective, practical means of preventing or reducing the amount of pollution generated by the project to a level compatible with Florida water quality standards found in chapter 17-3, Florida Administrative Code. The design of any stormwater best management practice shall be based on the seasonal high-water elevation determined by either the project geotechnical engineer, if the seasonal high-water elevation is estimated to be below ground, or the project biologist, if the seasonal high water elevation is determined to be above ground.  The design calculations for the selected stormwater best management practice shall use a design normal water elevation equal to the highest measured seasonal high-water elevation plus 6 inches.

…

- Design of water retention or detention structures and flow attenuation devices shall be subject to the approval of the CDE pursuant to the standards hereof. Detention structures shall be designed to release runoff to the downstream drainage system over a period of time so as not to exceed the capacity of the existing downstream system. Under no case shall open retention areas (ponds, etc.) have side slopes steeper than four horizontal to one vertical (4:1) to a depth of two feet of water at seasonal low pond elevation, at which point the side slope may be increased to two horizontal to one vertical (2:1). Retaining walls may be utilized to accommodate field conditions. In order to maintain good water quality in stormwater management detention ponds and maximize the provision of fish and wildlife habitat, stormwater management systems with permanently wet detention ponds should be designed, operated and maintained so as to resemble a natural pond to the greatest extent practical. A natural pond design should include: A littoral zone comprised of native emergent and submersed aquatic macrophytic vegetation; a deep open water limnetic zone free of rooted emergent and submersed vegetation; and, where feasible, an upland buffer of native trees, shrubs and under story vegetation in accordance with St. John's River Water Management District requirements. Stormwater ponds should be located internal to the project if possible.  If a stormwater pond is located adjacent to a perimeter property line and any portion of the adjacent property is lower than 1’ below the top of bank of the stormwater pond then the CDE may require a clay core, or similar device, to eliminate or reduce to historic levels the groundwater flow from the stormwater pond to the adjacent property.

…

    - For application for a standard development, the following additional performance standards shall be followed in the design of the project:

…

- Runoff computations. Runoff computations shall be based on the most critical situation (rainfall duration, distribution and antecedent soil moisture condition) and conform to acceptable engineering practices using rainfall data and other local information applicable to the affected area.  The tailwater elevation used in the design of the stormwater best management practice is to be based on a County approved basin study.  If a basin study has not been conducted and approved by the County than prior to preparing the stormwater best management practice design for the project, the engineer of record shall be responsible for coordinating with County Engineering staff to review and approve the tailwater information to be used in the calculations. Non-study based tailwater conditions are to surveyed by a licensed surveyor.  For the purposes of calculating the pre-development runoff for undeveloped property with multi-designation soils types, e.g. A/D or B/D soils, the curve number shall be based on the highest percolation rate soil, A or B, unless the project geotechnical engineer can prove to the satisfaction of the CDE that the curve number should be based on the lower percolating condition.

…

SECTION IV:  Authorizing Inclusion in Code. The provisions of this ordinance shall be included and incorporated into the Code of Ordinances of the County of Volusia, as additions or amendments thereto, and shall be appropriately renumbered to conform to the uniform numbering system of the code.

SECTION V:   Severability. Should any word, phrase, sentence, subsection, or section be held by a court of competent jurisdiction to be illegal, void, unenforceable, or unconstitutional, then that word, phrase, sentence, subsection, or section so held shall be severed from this ordinance and all other words, phrases, sentences, subsections, or sections shall remain in full force and effect.

SECTION VI:  Conflicting Ordinances. All ordinances, or part thereof, in conflict herewith are, to the extent of such conflict, repealed.

SECTION VII:  Effective Date. This ordinance shall take effect upon filing of a certified copy by e-mail with the Department of State.

FIRST READING, this ___ day of ___________, 2025.

UPON SECOND READING, BE IT ORDAINED, BY THE COUNTY COUNCIL OF VOLUSIA COUNTY, FLORIDA, IN OPEN MEETING DULY ASSEMBLED IN THE COUNTY COUNCIL CHAMBERS AT THE THOMAS C. KELLY ADMINISTRATION CENTER, 123 WEST INDIANA AVENUE, DELAND, FLORIDA, THIS DAY OF 2024 A.D.

COUNTY COUNCIL

ATTEST:						COUNTY OF VOLUSIA, FLORIDA

_____________________________		________________________________

George Recktenwald				Jeffrey S. Brower

County Manager					County Chair